# HOLOGRAPHIC AR GLASSES

Technical Reference — Target Specification, Derivations & Open Gaps

Prepared for specialist conversations — optical engineering, waveguides, panel suppliers, silicon architecture, investment

v19 · Two findings that predated this project's multi-seed and full-resolution standards were caught and corrected this revision, without a prompted audit. 10.3's long-standing "no benefit for sparse content" sequential-rendering conclusion — present in this document since its earliest surviving revision — is reversed: properly seed-tested (25 pairings), near and mid content show a small but consistently real advantage, never crossing zero. Separately, 10.8's content-adaptive plane-count candidate is retracted outright — a full-resolution retest flipped which content profile benefits more, the same jump-to-scale reversal pattern seen elsewhere in this document. A systematic audit of what else in this document predates these standards is flagged as still needed, not yet done. See 10.3, 10.8, 10.9, and Appendix A.13.

This document derives every target number in the system from first-principles physics and states every open gap with its magnitude. The posture is unchanged from v12–v18: surface what's unresolved before a specialist finds it, and treat every clean-looking number as a hypothesis until it's been actively attacked, not accepted because it looks reasonable. The numbers you need to walk into a room with are in the main body; the record of how each one was tested, broken, and in some cases fixed, is in the appendix for anyone who needs to audit or reproduce it.

# 1. Purpose & Posture

The goal of this document is to move a holographic AR glasses concept from a physics-grounded specification toward staged hardware validation. Four structural gaps (panel availability, compute power, color, eye tracking) are surfaced up front rather than discovered by a specialist mid-meeting. Investment and validation are staged so that later, more expensive commitments are gated on proof from earlier, cheaper ones.

The bar for this project is not "better than existing AR/VR." Every shipping and prototype device in Section 7 caps usable wear time at roughly an hour because of the vergence-accommodation conflict (VAC) — a genuine physiological mismatch between where the eyes converge and where the optics force them to focus, not a comfort or resolution problem that better engineering of the existing approach fixes. Varifocal and multifocal approaches reduce VAC's severity but don't eliminate it, and the actuation they require doesn't fit a glasses form factor. This architecture's target is a device worn all day, with the wearer's real surroundings passed through optically (not via camera feed), which is a different category than anything currently shipping.

# 2. The Governing Equation

Every spatial number in this system — pixel count, panel size, eyebox, field of view — is derived from a single conserved quantity: the space-bandwidth product (the coherent-optics analogue of étendue). It is conserved through any lossless passive optic. No waveguide material or index of refraction can increase it; a high-index material can only reduce how much of it is wasted.

N = 2 · W_eyebox · sin(FOV / 2) / λ

| Term | Meaning |
| --- | --- |
| N | Pixel count required, one axis |
| W_eyebox | Eyebox width — the window your pupil must sit inside |
| FOV | Full field of view, that axis |
| λ | Wavelength — worst case is the shortest (blue, ~450nm) |

Note: Pixel PITCH does not appear in this equation. Pitch sets physical panel size and the waveguide's required angular magnification — not pixel count. FOV is pixel-count-limited, not material-limited.

# 3. Target Specification

| Parameter | Target | Basis |
| --- | --- | --- |
| Panel resolution | 4,700 × 2,700 (~12.6 Mpix) | SBP invariant at λ=450nm worst case |
| Pixel pitch | 2.0 µm | Sets panel size; open — no confirmed supplier |
| Panel active area | 9.4mm × 5.4mm (~10.8mm diag.) | N × pitch |
| Native panel diffraction angle | ±6.46° | sin θ = λ / 2p |
| Required waveguide ang. magnification | ~3.87× | 25° needed ÷ 6.46° native |
| Eyebox, instantaneous | 2.5mm | ~daylight pupil diameter |
| Eyebox, effective | 5.0mm | 2×2 time-multiplexed |
| Field of view | 50° × 28° (~57° diagonal) | Design target, not physics-fixed |
| Composite frame rate | 360Hz (eyebox steering); ~1,080Hz-equivalent likely once color is included (Gap 9) | 90Hz perceptual × 4 (2×2 tiles) |
| MEMS settle time required | < ~1 ms | Within 2.78ms slot at 360Hz |
| CGH compute (current baseline) | ~1,291.7 TFLOP/s, 5-seed average (10.1), monochrome, cold-start | 552±198 FFTs/frame avg × 6.5 GFLOP × 360Hz — genuine seed variance, not noise |
| CGH ASIC power (current baseline) | ~199W ± 71 measured, 5-seed average (10.1), monochrome (range 130–282W); ~597W estimated with color (Gap 9, not yet jointly measured) | 1,291.7 TFLOP/s ÷ 6.5 TOPS/W (BFP16) |
| Power budget target | ≤ 500 mW — ~397× over at current monochrome average (range 260×–564×); ~1,194× with color estimate | See 10.1 and Gap 2 |
| Laser optical output | ~18 mW | 3000 nits target, 0.2% system efficiency |
| Thermal sustainable budget | ~324–540 mW | 10.8cm² contact × 30–50 mW/cm² |
| Battery capacity (est.) | ~1–2 Wh | Glasses-form estimate |
| Continuous runtime | ~46–92 min at the original ~1.3W estimate; ~18–36s at the measured ~199W average baseline (range ~13–55s across seed variance, 10.1) | Forces duty-cycled or content-adaptive use |
| Gaze tracking requirement | ±0.5mm, 200Hz, <5ms latency | Tabled — safety-critical dependency |

Note: Every figure in this table with a citation to Section 10 is a live measurement, not a closed spec — treat this table as current-best-known, not final. The compute/power figures were rebaselined this revision to a 5-seed average after a single-seed figure proved unstable; see 10.1. A preliminary, algorithm-level finding (10.6) suggests INT8 precision may match BFP16 quality — if confirmed on real hardware, this would cut the power figures above roughly 3× (to ~65W, ~129× over budget). Not yet adopted into the table above pending hardware or specialist confirmation.

# 4. Derivation Walkthrough

## 4.1 Panel resolution

Solved for N at λ=450nm (worst case), W_eyebox=2.5mm, half-FOV=25° (horizontal) and 14° (vertical).

N_x = 4,696   N_y = 2,688   →  ~12.6 Mpix total

Note: Rounds to 4,700 × 2,700 for physical layout purposes.

## 4.2 Panel size & waveguide angular magnification

Panel size follows directly: N × pitch. The panel's own native diffraction angle falls well short of the required FOV half-angle.

sin θ_max = λ / 2p  →  θ_max = 6.46° at p = 2.0µm.  Required magnification = 25° / 6.46° ≈ 3.87×

Note: This magnification must come from optical power built into the waveguide combiner while preserving coherent phase — see Gap 3. No vendor has confirmed this.

## 4.3 Eyebox multiplexing & frame rate

A 2×2 grid of 2.5mm sub-positions, cycled fast enough to look continuous, covers a 5mm effective eyebox without growing pixel count: 90Hz (perceptual) × 4 (2×2 positions) = 360Hz composite.

Note: MEMS steering must settle within ~1ms of the 2.78ms slot, repeatably, for years of daily use — unverified against any named component.

## 4.4 CGH compute — original closed-form estimate

8 FFTs to build a layered 3D target + 6 FFTs for iterative phase retrieval = 14 FFTs/frame, at ~1.49 GFLOP per 2D FFT (5·N·log₂N): 14 × 1.49 GFLOP = 20.8 GFLOP/frame × 360Hz ≈ 7.5 TFLOP/s sustained.

Note: This closed-form estimate is kept for traceability — it's a clean formula the measured figures in Section 10 are not. It significantly undershoots the measured cost: both the FFT-count assumption (14) and the per-FFT GFLOP assumption (1.49, unpadded) were found too low. See 10.1 for the current measured figure and Appendix A.1 for how the estimate evolved.

## 4.5 Power — the precision ceiling

FFTs cannot run at INT8 (20 TOPS/W) without corrupting the wave computation — dynamic range collapses (this is a stated assumption, not yet independently tested against real content — see 10.6 for a proposed test). 16-bit block floating point (BFP16) is required, at a realistic ~6.5 TOPS/W.

7.5 TFLOP/s ÷ 6.5 TOPS/W ≈ 1,154 mW

Note: ~2.3× over the 500mW budget at this derivation's inputs alone. The measured end-to-end figure (10.1) is far larger — see Gap 2.

## 4.6 Brightness / laser output

3000 nits over a 3mm pupil across the target FOV, converted to lumens, then optical watts, then inflated for a pessimistic 0.2% end-to-end system efficiency: 9.0mlm ÷ 250 lm/W ÷ 0.2% ≈ 18 mW laser output.

Note: Brightness is not a bottleneck for this architecture — under 2% of the compute chip's power draw. Eye-safety interlock on modulation failure is still a hard requirement.

## 4.7 Thermal budget

Temple-arm skin contact (~10.8cm²) is the only heat exit — no airflow. Comfort ceiling ~30–50 mW/cm² continuous: 10.8cm² × (30 to 50) mW/cm² ≈ 324–540 mW sustainable.

Note: The compute chip alone, even at the original 1,154mW estimate, already exceeds this ceiling — a second, independent constraint from the power budget, not the same problem twice.

## 4.8 Battery life

Glasses-scale battery estimated at 1–2 Wh (vs. ~10–15 Wh for a phone), against a ~1.3W total system draw including driver overhead: 1–2 Wh ÷ 1.3 W ≈ 46–92 minutes continuous.

Note: Confirms duty-cycled display operation is required for two independent reasons (thermal + battery). See 10.5 for a preliminary finding on reducing average — not peak — compute via temporal reuse.

# 5. Honest Gaps — Lead With These

| Gap | Magnitude | Status |
| --- | --- | --- |
| 1. Panel | No commercial phase modulator hits 4,700×2,700 @ 2µm pitch @ 360Hz+ with full 2π phase depth | Blocking. Commercial monochrome ceiling is 60Hz (GAEA-2.1's 180Hz is a Color Field Sequential field rate, not monochrome) — ~6× below the 360Hz target. Ferroelectric LC shows promise (>2,000Hz research-grade) but isn't a commercial part yet. Color alone likely needs ~3 sub-frames/perceptual frame (Gap 9) — combined with 4× eyebox steering, a ~12-sub-frame, ~1,080Hz-equivalent requirement before depth rendering is even considered. |
| 2. Compute power | ~199W ± 71 measured, 5-seed average (10.1, monochrome), range 130–282W; ~597W estimated with color vs. 500mW budget — ~397× over on average (range 260×–564×); ~1,194× with color estimate | A full-resolution, real-content, true-convergence measurement (10.1) found 552 FFTs/frame on average — but a single-seed version of this same measurement (434 FFTs, 156W, 312×) was found to vary genuinely with random seed, not just measurement noise, after a materially improved plateau-detector failed to close the gap (Appendix A.7). Closing this gap requires a different architecture or algorithm, not just a faster implementation. Two preliminary, unconfirmed levers: temporal reuse between similar frames may reduce average (not peak) compute (10.5); INT8 precision may match BFP16 quality algorithmically, a possible ~3× power reduction if confirmed on real hardware (10.6). |
| 3. Eye tracking | ±0.5mm @ 200Hz, <5ms latency, rim/bridge form factor | Safety-critical — tabled; two immature candidate approaches |
| 4. Waveguide angular magnification | ~3.87× required while preserving coherent phase across pupil replication | Unquantified — needs vendor engagement |
| 5. Thermal ceiling | ~324–540mW sustainable vs. ~199W average measured — ~368–614× over, not ~2× (range ~240×–870× across measured seed variance) | Forces duty-cycling — product-level, not just engineering |
| 6. Battery life | ~18–36 seconds continuous at the measured ~199W average baseline (range ~13–55s across seed variance); ~46–92 min only at the superseded ~1.3W estimate | Same root cause as Gap 5. Temporal reuse (10.5) may improve average runtime without changing this worst-case figure |
| 7. Speckle / multi-plane quality ceiling | Coherent illumination + reduced independent modes; compounded by simultaneous multi-plane phase encoding | Representational, not algorithmic — confirmed by three independent lines of evidence (10.2). Sequential depth-plane rendering is a confirmed mitigation for all content types once properly seed-tested (10.3) — small but real for sparse content (near/mid, ~0.3–2 dB), larger for dense content (far, ~2.8–4.4 dB) — superseding an earlier single-seed finding that reported no benefit for sparse content |
| 8. Industrial design | Optical engine implies ~9–10mm localized temple bulge vs. ~3–4mm standard | Tabled for now — real constraint, deferred |
| 9. Color / chromatic dispersion | Native 6.8 dB, naive reuse 5.8 dB (−1.0 dB), joint solve 6.3 dB (−0.5 dB short of native), corrected metric, full resolution | Characterized (10.4). Joint multi-color solve is a candidate this document proposes from simulation, not an observed commercial technique — worth raising with a panel supplier directly |

# 6. Structural Advantages — Worth Stating Alongside the Gaps

- Prescription correction is free: defocus, astigmatism, and higher-order aberrations are additional phase terms in the same computation already being run — no inserts, one SKU fits every eye.

- Accommodation is correct by construction: VAC is eliminated because the eye performs genuine per-pixel focusing — a property no shipped or demoed competitor has.

- Brightness is not a bottleneck: ~18mW of required laser output is under 2% of the compute chip's power draw.

- Per-unit waveguide aberrations can be calibrated out in software rather than tightened in manufacturing tolerance.

# 7. Comparative Landscape

None of the three reference products below perform true computer-generated holography — all render a conventional fixed-focus image, even when marketing language uses "holographic" to describe the waveguide's manufacturing method rather than the display's compute method.

| Product | Display / Waveguide | FOV | Resolution | Focus / VAC |
| --- | --- | --- | --- | --- |
| Ray-Ban Display | LCoS + geometric (Lumus) waveguide, monocular | 20° | 600×600, 42 PPD | Flat, fixed — avoids triggering VAC via single-eye static UI |
| HoloLens 2 | Laser + MEMS scan, diffractive waveguide | ~52° diag. | 720p-class | Fixed ~2m focus — confirmed VAC |
| Meta Orion (prototype) | microLED, silicon-carbide diffractive waveguide | 70° | 13 PPD (low) | Fixed focus — VAC present, not solved |
| This design (target) | Phase SLM (CGH), phase-preserving waveguide, binocular | 50° × 28° | ~12.6 Mpix/eye | True per-pixel accommodation — VAC eliminated by design |

Note: The last row's resolution is the SLM's phase-pixel count, not perceived image resolution — not directly comparable to the others. Section 10.2's finding (~11–14 dB PSNR, representationally limited) shows delivered quality today is well below what the raw pixel count implies.

# 8. Questions By Specialist

## Optical engineer / waveguide house

- Can your combiner supply ~3.87× angular magnification while preserving coherent phase across a replicated exit pupil?

- What's your measured phase distortion across pupil replication, and do you support storing per-unit calibration data?

- Can optical power be placed in the outcoupler itself, folding the Fourier lens into the combiner?

- What end-to-end efficiency do you deliver into a 2.5–5mm gaze-steered eyebox at a 3000-nit target?

## Panel supplier

- What's your fastest full-2π phase modulator under 3µm pitch, and what's the phase quantization at that speed? Commercial phase-only LCOS today tops out around 60Hz for genuine monochrome addressing.

- Can you support 360Hz+ full-frame updates? If not natively, what's the NRE and timeline for a custom backplane at 4,700×2,700, 2µm pitch?

- Ferroelectric LC vs. custom backplane vs. relaxing the resolution target — which do you recommend given the eyebox/FOV trade?

- For color: does a Color Field Sequential mode fit this architecture, and what's the real frame-rate cost at full resolution?

## Silicon architect

- What's the achievable efficiency (TOPS/W) for a hardwired streaming radix-4 FFT block, 16-bit block floating point, at 16nm and at 3nm?

- A full-resolution, real-content, true-convergence measurement, averaged across 5 random seeds after single-seed measurements proved unstable, puts sustained compute at ~1,291.7 TFLOP/s, implying ~199W average at today's realistic BFP16 ceiling — ~397× the 500mW budget on average, before color (Gap 9) multiplies it further. Preliminary quantization-emulation testing (not yet hardware-confirmed) suggests INT8 precision may reach the same quality as BFP16 — if true, that's ~65W, ~129× over budget, via 20 TOPS/W instead of 6.5. What closes the remaining gap: hardware confirmation of INT8 viability, analog/in-memory compute, a different node — or does the algorithm itself need to change?

- What does an FPGA proof of the dataflow cost and how long does it take, before committing to test silicon?

## Investor

- Sequencing is staged, not a single ask: FPGA validation → 16nm test silicon → 3nm production, each gated on proof from the prior stage.

- Lead with the staged number, not the terminal one — the 3nm figure alone makes the ask sound unbounded when it is actually bounded per stage.

## Tabled for a later pass (eye tracking, industrial design)

- Eye tracking: oblique IR camera vs. MEMS-scanned photodiode vs. relaying the eye image out through the display waveguide itself — genuinely open.

- Industrial design: how to house a ~9–10mm localized bulge near the hinge without reading as bulky — deferred by choice.

# 9. Staged Validation & Investment Sequence

| Stage | Goal | Rough Cost | Gates |
| --- | --- | --- | --- |
| FPGA dataflow validation | Prove the CGH pipeline architecture and timing | Low hundreds of thousands | Gates test silicon commitment |
| 16nm test silicon | Validate FFT block efficiency, real power draw, node choice | $10–20M | Gates production silicon commitment |
| 3nm production silicon | Ship at volume | $100M+ | Only after architecture is proven, not before |

Note: Lead with this staged framing in investor conversations rather than the terminal 3nm number alone.

# 10. Software Validation — Current Findings

Before engaging optical, panel, and silicon specialists, a software simulation of the CGH phase-retrieval pipeline was built to test the two most specialist-independent assumptions in this document: the compute estimate and achievable image quality. Testing began on CPU/small-scale GPU, then scaled to the full 4,700×2,700 target resolution with real rendered content (a UI icon, translated caption text, a photo-like scene) on a consumer RTX 3060. Two caveats remain open throughout: no waveguide, eye, or aberration model is included, and the quality bar used for comparison (10.6) is a placeholder pending human perceptual testing or an optical engineer's judgment.

This section states current-best-known findings plainly. The full record of how each one was tested, what broke, and what was fixed — including two measurement bugs found and corrected along the way — is preserved in Appendix A for anyone auditing or reproducing these numbers.

## 10.1 Compute estimate — current measured baseline (5-seed average)

A genuine end-to-end convergence measurement — full target resolution (4,700×2,700), real content, run to true convergence with plateau auto-detected rather than assumed — initially produced a single-seed figure of 434 FFTs/frame, ~156W, ~312× over budget. That figure superseded an earlier measurement (696 FFTs/frame, ~250W, ~500×) which could not be reproduced from any surviving code state — not an error, just never committed before being overwritten (Appendix A.4).

The single-seed figure was then tested for seed sensitivity, on the reasoning that a plateau-detected convergence point sitting anywhere in a flat, noisy region of the curve might not be reliable from one random draw:

| Seed | Plateau iteration | Final quality (dB) | FFTs/frame | Power | × budget |
| --- | --- | --- | --- | --- | --- |
| 0 | 36 | 11.85 | 434 | 156W | 312× |
| 1 | 65 | 11.89 | 784 | 282W | 564× |
| 2 | 62 | 11.70 | 748 | 269W | 538× |
| 3 | 36 | 12.00 | 434 | 156W | 312× |
| 4 | 30 | 11.47 | 362 | 130W | 260× |

552 ± 198 FFTs/frame  ×  6.5 GFLOP/FFT  ×  360Hz  =  ~1,291.7 TFLOP/s average  →  ~199W ± 71 average  →  ~397× over the 500mW budget on average (range 260×–564×)

Note: Final quality is tight across seeds (11.47–12.00 dB, 0.53 dB spread) while iterations-to-plateau vary widely (30–65) — confirmed as genuine algorithmic variance in how different random starting phases converge, not a plateau-detection artifact: a materially improved detection criterion (sustained quality hold against an averaged reference, replacing a single-threshold crossing) was tested directly against these same five convergence histories and reduced the spread by less than 3% (35 iterations → 34). See Appendix A.7 for the full comparison.

Note: Single-seed convergence measurements are no longer treated as sufficient for any figure in this document. Every downstream reference to compute/power (Sections 3, 5, 8, 11) uses this 5-seed average.

## 10.2 Image quality and speckle ceiling (Gap 7)

Reconstructed images plateau at roughly 11–14 dB PSNR (corrected metric — see Appendix A.2) — visibly grainy, consistent with coherent speckle. Three independent lines of evidence converge on the same conclusion: the limitation is representational, not a search or tuning problem.

- Averaging multiple independent phase-retrieval solutions produces only a small quality gain, indicating a meaningful share of the error is systematic, not random noise that would average out.

- Quality degrades monotonically with simultaneous depth-plane count: 14.6 dB (1 plane) → 11.9 dB (2 planes) → 11.5 dB (3 planes), corrected metric.

- A learning-rate sweep across four materially different optimization strategies produced under 0.6 dB of spread — ruling out optimizer tuning as the limiting factor.

- A follow-up test comparing GS, direct pixel-wise gradient descent, and a CNN-parameterized ("neural-prior") phase representation at equal step budget found no benefit from the network's structural bias (11.07 dB vs. SGD's 13.83 dB) — a fourth, architecturally distinct line of evidence pointing the same direction.

Note: One phase-only pattern cannot fully satisfy multiple simultaneous depth-plane constraints — this is a structural finding, not a known unknown. Part of the problem is addressable by architecture rather than optics alone; see 10.3.

## 10.3 Depth-plane rendering: sequential vs. simultaneous

Sequential rendering (solving and displaying each depth plane's phase pattern separately, at 3× the sub-frame cost) shows a real quality advantage at small scale. The original conclusion at target resolution — reported in every prior revision of this document — was that this advantage survives for dense content but disappears entirely for sparse content (icons, text). That conclusion is superseded this revision: it was based on a single seed pairing per condition, never tested against the multi-seed standard this document has applied to every other convergence-speed or quality comparison since 10.7/A.9.

Retested with 5 seeds per condition (25 independent seed pairings total, simultaneous vs. sequential) at target resolution:

| Plane | Simultaneous avg | Sequential avg | Delta range across 25 seed pairings |
| --- | --- | --- | --- |
| Near (sparse) | 7.47 | 8.79 | +0.87 to +2.06 dB |
| Mid (sparse) | 10.53 | 11.21 | +0.30 to +1.21 dB |
| Far (dense) | 11.23 | 14.70 | +2.81 to +4.42 dB |

Note: Every one of the 25 seed pairings, for every plane, shows a positive delta — sequential never loses to simultaneous, anywhere in this sample. The original single-seed test's near/mid deltas were genuinely small enough (sub-1–2 dB) to be indistinguishable from seed-to-seed noise in one comparison; properly seed-tested, the sign never actually crosses zero. This is a real, if modest, effect for sparse content, not the "no benefit" this document previously reported.

Note: Methodology caveat: this retest used the project's amplitude-vs-amplitude convergence metric (matching GS's own internal optimization target, consistent with the experiment scripts used since 10.7), not confirmed to be on the same absolute scale as the psnr_intensity-corrected metric behind the ~1.9–2.1 dB dense-content figure quoted in earlier revisions and Gap 7. The qualitative finding — near and mid show a real, consistently positive effect, not zero — is trustworthy on its own terms; the exact dB magnitudes above should not be assumed directly comparable to previously published corrected-metric figures without confirming consistent methodology first.

Practical read: near/mid's advantage (roughly 0.3–2 dB) is real but small next to far's (2.8–4.4 dB), and Gap 1 already shows no commercial panel meets even the 360Hz base requirement, let alone the 1,080Hz-equivalent a 3× sub-frame cost would demand. The likely product recommendation — don't prioritize sequential rendering for near/mid content — probably still holds, but for a different and more honest reason than this document previously stated: a real, measured, small benefit judged not worth a 3× frame-rate cost, not the absence of any benefit at all.

Note: This reversal was caught by applying the multi-seed standard retroactively to an old finding that predated it, not by a new test design. It raises a real open question about what else in this document was measured before that standard existed and hasn't been re-checked since — flagged in 10.9, not yet resolved by a systematic audit.

## 10.4 Color / chromatic dispersion (Gap 9)

A phase pattern solved for one wavelength does not fully transfer to others. At full target resolution, across all three depth planes, corrected metric:

| Approach | Quality (avg. red+blue) | Cost |
| --- | --- | --- |
| Native (separate solve per color) | 6.8 dB | 3 sub-frames — matches HOLOEYE GAEA-2.1's Color Field Sequential practice |
| Naive reuse (green phase, unmodified) | 5.8 dB (−1.0 dB) | 1 sub-frame — effectively free |
| Joint (single pattern, 9 constraints at once) | 6.3 dB (−0.5 dB short of native) | 1 sub-frame at same total FFT cost as native — proposed design point, not commercially observed |

Note: Joint solve was measured at a fixed 16-iteration cutoff, which 10.1's convergence work found premature for true convergence — a 9-constraint joint solve may need more iterations than a 3-constraint native one, in which case joint may not be compute-neutral as tested. Treat as a candidate, not a confirmed design point.

## 10.5 Temporal warm-starting across frames

Every measurement above solves each frame from scratch, starting from random phase. Real content is video: adjacent frames at 360Hz are often similar. This section tests whether seeding each frame's phase-retrieval optimization from the previous frame's converged phase — instead of random noise — reduces iterations needed to reach threshold quality.

Tested against a synthetic panning sequence (no real motion-capture content used yet) at both 512×512 and the full 4,700×2,700 target resolution:

| Test | 512×512 | Full res (4,700×2,700), real content |
| --- | --- | --- |
| Steady pan, small shift (~0.6% of frame) | ~14.9× | ~8–9× (3-seed range: 7.67×–9.33×) |
| Steady pan, large shift (~12% of frame) | ~8.3× | ~6.0× |

Note: The steady-pan speedup survives the jump to target resolution — unlike the sequential-rendering finding in 10.3, which vanished entirely at this same resolution scale-up. It rests on one shift magnitude at each scale, though — not yet the breadth of testing behind 10.2–10.4.

Note: Re-tested across 3 seeds after 10.1's compute baseline revealed seed-dependent fragility elsewhere in this project's convergence measurements: 9.33×, 9.33×, 7.67× — a ~19% spread, far tighter than the ~76% swing seen in the single-seed compute baseline before averaging. This is structural, not luck: warm-start's threshold crossing usually happens on the very first iteration, not in a noisy plateau region, so it isn't exposed to the same fragility. Reported as ~8–9× going forward, not a single point.

### Scene-cut robustness — retested, and the original finding did not hold

An initial single-pair test (10.5, earlier revisions) found severe negative transfer at 512×512 (warm-start never converged within a 30-iteration budget after an abrupt content change) that appeared to partially reverse at full resolution. Per the standing rule established in 10.7/A.9, neither of those results had been checked against true, plateau-verified convergence — both used a small, fixed iteration budget. Five scene-cut pairs, at two severities (moderate: shifted content; severe: fully permuted content-to-depth-plane assignment), were retested at both scales with a generous, plateau-auto-detected budget instead:

| Scale | Negative transfer | Pairs where warm-start beat cold-start average |
| --- | --- | --- |
| 512×512 | 0 of 5 pairs | 4 of 5 |
| 4,700×2,700 (full res) | 0 of 5 pairs | 2 of 5 |

Note: The original catastrophic-failure finding does not reproduce under correct methodology, at either scale, across five pairs including several more severe (full content-to-depth-plane permutation, not just a spatial shift) than the original single test case. This is the same failure mode as the donor-seeding reversal in 10.7, pointed in the opposite direction: there, a small budget manufactured a fake win; here, a small budget manufactured a fake catastrophe. Both are now retired as fixed-budget artifacts, and both reinforce the same standing rule from independent directions.

Note: The speedup side of the finding is weaker than it looked at small scale, though, in the same shrinking-at-scale pattern seen elsewhere in this document (10.3): 4 of 5 pairs beat cold-start at 512×512, but only 2 of 5 did at full resolution — the rest converged normally, just without a clear speed advantage. The honest summary is that warm-starting through a scene cut is usually roughly neutral to mildly helpful, essentially never catastrophic, at least across the pairs tested here.

### Independent-content validation — closing the shared-source-content gap

Every result above — steady-pan, scene-cuts — was built from one base scene, varied only by shift or plane permutation. This retests both against genuinely independent, procedurally-generated content (a different random icon, word, and Gaussian-blob "photo" scene per trial, not derived from a shared source) and a smoother parallax-motion model (per-plane random-walk velocity, nearer planes moving faster than farther ones, matching real motion parallax) instead of constant-velocity shift. This is still synthetic content, not real motion-capture or recorded footage — a step up in realism, not a claim of product-content validation.

Getting a trustworthy result here required catching a second, different kind of methodology artifact than 10.7/A.9's fixed-budget problem. The initial run reported warm-start plateauing at exactly iteration 5 for nearly every frame across all three motion sequences tested — a suspiciously uniform number. Investigation found the cause: the project's local-window plateau detector (used by default since 10.7) checks only whether a short recent window is locally flat, and 5 was the earliest iteration it was structurally capable of returning at all — a detector floor, not a genuine finding. Cross-checked against a different, final-value-anchored detector (confirming a candidate plateau's value actually matches where the curve eventually settles, not just that it looks flat nearby) and a manual seven-frame spot-check:

| Frame | Local-window detector | Final-value-anchored detector | Verdict |
| --- | --- | --- | --- |
| 1 | 9 | 154 | False positive — curve was still climbing substantially |
| 2 | 1 | 1 | Real — reused-phase quality within 0.07 dB of eventual final value |
| 3 | 1 | 1 | Real — within 0.08 dB |
| 4 | 1 | 1 | Borderline — 0.11 dB gap |
| 5 | 1 | 1 | Real — within 0.05 dB |
| 6 | 1 | 114 | False positive — curve was still climbing substantially |
| 7 | 117 | 121 | Real, both detectors agree |

Note: Two of seven spot-checked frames were false positives from the local-window detector, not just the suspicious uniform-5 case that triggered the investigation. This is a different failure mode than 10.7/A.9's fixed-budget problem — the budget here was already generous — but the same underlying lesson: a plateau-detection method that only checks local flatness, without confirming the flat region matches the run's true eventual value, can silently report false convergence even with plenty of iterations available. The local-window detector is retired as this project's default going forward; the final-value-anchored method is the standard. See Appendix A.11 for the full record.

Results, using the final-value-anchored detector throughout:

| Independent scene-cut pairs | Warm-start iterations | Cold-start average | Result |
| --- | --- | --- | --- |
| 0 → 1 | 3 | 87.7 | Beats |
| 1 → 2 | 84 | 125.3 | Beats |
| 2 → 3 | 21 | 80.7 | Beats |
| 3 → 0 | 77 | 80.7 | Beats (barely) |

4 of 4 independent scene-cut pairs beat cold-start; 0 of 4 showed negative transfer — consistent with, and now independently confirming, the shift/permutation-based result in the section above. Under parallax motion, across 3 independently-seeded sequences (21 real frame-to-frame comparisons, excluding each sequence's shared first frame): 17 of 21 comparisons favored warm-start (81%), with per-sequence win rates of 4/7, 7/7, and 6/7 — genuinely favorable on balance, not a uniform win, and with real, currently unexplained variability in how consistently it helps from one motion sequence to the next.

Note: This validates rather than contradicts every finding in this section: warm-starting helps on both similar and genuinely independent content, scene cuts do not cause catastrophic failure even between fully unrelated scenes, and the honest picture is "usually favorable, sometimes dramatically, occasionally a net cost" rather than a uniform win in either direction. The seed-to-seed spread in win rate (4/7 to 7/7) is noted but not investigated further here.

Read the steady-pan result as a duty-cycle finding, not a peak-compute reduction: average power draw across a real usage session could be substantially lower than the steady-state figure in 10.1 implies, for content that doesn't change abruptly frame to frame. The scene-cut retest, unlike the earlier revision's conclusion, does not currently indicate that a dedicated cut-detection fallback (reverting to cold-start rather than warm-starting from an unrelated prior frame) is necessary — the worst case observed across all trials, including independent content, was a normal, converging solve, not a failure to converge.

## 10.6 Precision: does INT8 corrupt the computation? (tests the assumption in 4.5)

Section 4.5 assumed, without testing, that INT8 precision (20 TOPS/W) corrupts the CGH wave computation and that BFP16 (6.5 TOPS/W) is required. This was tested via quantization emulation — rounding the field to N-bit fixed-point precision at each point in the GS loop a real ASIC datapath would actually round (after each forward propagate, after the amplitude constraint, after each backward propagate), then continuing the surrounding arithmetic in float32. This approximates the cumulative effect of running at N bits but does not model rounding error accumulating inside a single FFT's internal butterfly stages — see Appendix A.8.

| Bits | Final dB (full resolution, true convergence) | Δ vs. float32 |
| --- | --- | --- |
| 4-bit | 10.79 | −1.06 |
| 8-bit | 11.86 | +0.01 |
| 16-bit | 11.94 | +0.09 |
| float32 (baseline) | 11.85 | — |

8-bit (INT8-equivalent) quality matches float32 within noise — confirmed at full target resolution, at true convergence, and held up under three direct attempts to break it: replacing an oracle quantization scale with a realistic static one, retesting at full resolution and true convergence rather than a small fixed-budget test, and checking an apparent "INT8 needs more iterations" side-finding across multiple seeds, which was retracted once it became clear the effect was the same seed-dependent plateau variance found in 10.1, not an INT8-specific penalty. 4-bit shows real, mechanistically sensible degradation — the quantization step is coarse enough that the algorithm exhausts what the noise floor allows within a few iterations and plateaus early. See Appendix A.8 for the full stress-testing sequence.

Note: This is algorithm-level evidence, not a hardware-validated result — see the methodology caveat above. If confirmed on real hardware, this is a direct ~3× lever on Gap 2: 20 TOPS/W instead of 6.5 would bring the current average baseline from ~199W to roughly ~65W (~129× over budget instead of ~397×). Not yet adopted as this document's headline power figure; the BFP16-based baseline in 10.1 is used throughout until independently confirmed by a silicon architect or FPGA test.

## 10.7 Initialization strategy — three approaches tested, all negative at true convergence

10.1/Appendix A.7 found that random-phase initialization causes genuine iteration-count variance (30–65 iterations across seeds) despite consistent final quality — some random starting points have a short path to the target, others a long one, with no way to know which in advance. Three deterministic (seed-independent) alternatives were tested to see whether a smarter starting point avoids gambling on the random draw.

| Approach | Mechanism | Result |
| --- | --- | --- |
| Flat-phase backprop | Average the target amplitudes' backward propagation with zero phase, once, before any GS iterations — reuses math already inside the GS loop itself | Underperformed even a naive zero-phase baseline (25 vs. 23 iterations to plateau, lower final quality) — a structured-but-wrong guess, worse than no information at all |
| Low-res coarse-to-fine seeding | Solve a fast low-resolution version of the same target first, upsample the result as the full-resolution initial phase | Did not beat plain random on total FFT cost at either budget tested (10 or 30 low-res iterations); a partially-converged low-res guess introduced a new failure mode — committing the full-resolution solve to worse structure than either a fully-converged guess or a fresh random start |
| Cross-session donor-seeding | Reuse a phase pattern already converged from unrelated (donor) content as a fixed, reusable default initializer, tested in both content directions | Looked like a real win at a fixed 80-iteration budget (+24% to +64% faster) — reversed on proper testing: see below |

The donor-seeding result is the most important of the three, not for the technique itself but for what testing it properly exposed. At a fixed 80-iteration budget:

| Direction | Random avg iters (80-iter budget) | Donor avg iters (80-iter budget) | Apparent speedup |
| --- | --- | --- | --- |
| A: realistic donor → synthetic target | 26.8 | 20.3 | 24% faster |
| B: synthetic donor → realistic target | 50.2 | 18.2 | 64% faster |

Rerun to true, verified convergence (200 iterations, plateau confirmed on both conditions in both directions) instead of a fixed budget:

| Direction | Random avg iters (true convergence) | Donor avg iters (true convergence) | Real effect |
| --- | --- | --- | --- |
| A: realistic donor → synthetic target | 40.2 (11.18 dB) | 41.5 (11.19 dB) | None — the 24% speedup was entirely a budget artifact |
| B: synthetic donor → realistic target | 94.2 (12.08 dB) | 108.7 (11.90 dB) | Reversed — donor-seeding needs 15% more iterations and costs ~0.18 dB, not a win at all |

Note: The mechanism in Direction B: a donor phase built for unrelated content isn't neutral — it appears to send the solve down a longer, more circuitous path before settling, the same structured-but-wrong pattern seen in the flat-phase backprop result above, not a coincidence. A biased starting point can cost more than starting from nothing, even when it looks like a head start within a limited budget.

Note: All three results point the same direction: none of the initialization strategies tested reduces or avoids the iteration-count variance characterized in 10.1/A.7. That variance currently looks like a structural property of this algorithm's search landscape, addressed only by seed-averaging (10.1), not by any starting-point strategy tried so far. The backprop and low-res results above predate this investigation's discovery of the convergence-budget artifact and have not been re-verified against it — flagged as an open caveat, not assumed safe, per the same standing rule this section establishes. See Appendix A.9 for the full record.

## 10.8 Two-plane dropoff — compute/quality tradeoff of removing the middle depth plane

This does not answer whether 2 planes are perceptually sufficient — that remains a question for a human or an optical engineer, not further simulation (see 10.9). What it measures instead: the compute and quality tradeoff of dropping the middle depth plane from the GS solve entirely, and a physical (not perceptual) proxy — when the solve is no longer told to reproduce content at the middle depth, does the resulting phase pattern still produce something close to that content there anyway, since a phase-only hologram isn't blind at unsolved depths?

Tested across two content profiles differing in how much is actually happening at the middle depth: CLOSEUP (sparse mid content, masked to a small region per the sparse-content methodology in 10.6/A.6) and LAYERED (dense, photo-like mid content filling the frame). Both propagate the 2-plane (near/far only) solution to the dropped mid depth and compare against the real mid target, the 3-plane baseline's actual solved mid quality (the ceiling), and a blank field (the floor).

The initial, raw-dB result came back opposite to the working hypothesis — CLOSEUP (sparse mid) showed a larger gap-to-ceiling than LAYERED (dense mid), the reverse of "less there to lose":

| Profile | Ceiling (3-plane) | Proxy (2-plane, unsolicited) | Floor (blank) | Gap to ceiling | FFTs saved |
| --- | --- | --- | --- | --- | --- |
| CLOSEUP (sparse mid) | 8.94 dB | 5.12 dB | 2.71 dB | 3.82 dB | +32.9% |
| LAYERED (dense mid) | 6.55 dB | 3.84 dB | 2.90 dB | 2.71 dB | +20.1% |

Note: Investigated rather than accepted at face value. Two genuine confounds were found: CLOSEUP's sparse mid content was correctly masked to a small region (per the established sparse-content methodology) while LAYERED's dense mid was measured over the full frame — different measurement bases, not a fair comparison. Separately, LAYERED's own 3-plane ceiling is inherently lower than CLOSEUP's (busy, textured content is harder to reconstruct at all, consistent with this document's established pattern elsewhere), leaving it less floor-to-ceiling range to begin with regardless of how much real content the dropped depth carried.

A normalized metric — the fraction of each profile's own floor-to-ceiling range recovered for free — controls for both confounds and reverses the finding back to the original hypothesis:

| Profile | Fraction of range recovered for free | FFTs saved |
| --- | --- | --- |
| CLOSEUP (sparse mid) | 38.7% | +32.9% |
| LAYERED (dense mid) | 25.8% | +20.1% |

Note: Sparse-at-the-dropped-depth content both survives more (38.7% vs. 25.8% of its own range recovered without being solved for) and saves more compute (32.9% vs. 20.1% fewer FFTs/frame) when the middle plane is dropped — a coherent, physically sensible pattern: less to solve for, less to lose, at 512×512. The normalized metric is a stated choice, not neutral ground truth — it weights each profile's dB range equally per percentage point, which isn't obviously the right assumption if two scenes have very different absolute ranges. Both raw and normalized numbers are reported together for this reason, per Appendix A.12.

This suggested a real design candidate beyond a fixed plane-count choice: a content-adaptive plane count, solving 2 planes when the middle depth is sparse and 3 when it isn't. It does not survive a full-resolution retest, and the finding is retracted, not merely weakened:

| Profile | Fraction recovered — 512×512 | Fraction recovered — full resolution (4,700×2,700) |
| --- | --- | --- |
| CLOSEUP (sparse mid) | 38.7% | 21.8% |
| LAYERED (dense mid) | 25.8% | 28.4% |

Note: The sign flips, not just the margin: CLOSEUP recovers more at small scale, LAYERED recovers more at full resolution — the same jump-to-target-resolution reversal pattern already seen elsewhere in this document (10.3, donor-seeding in A.9, scene-cut negative transfer in A.10), now confirmed a fourth time. The specific claim that content sparsity at the dropped depth predicts how much survives is retracted; it should not be treated as a supported finding. What does survive at both scales: dropping mid saves real compute either way (~20–33% fewer FFTs/frame across profiles and scales) and recovers meaningfully more than a blank floor in every case tested — dropping mid is a real, no-free-lunch-but-not-total-loss lever, just not one that content type can currently predict the size of. See Appendix A.12 for the full record.

## 10.9 Open items & next steps

- PSNR target: still a placeholder with no derivation behind it — the one item that needs human perceptual testing or an optical engineer's judgment rather than further simulation. This is the natural handoff point to specialist input.

- A border/grid artifact in full-resolution reconstructions remains mechanistically unexplained (deprioritized, non-blocking — see Appendix A.3). A similar unexplained non-monotonic pattern showed up in the temporal warm-start shift-magnitude sweep (10.5); the leading hypothesis (padding wraparound) was tested and ruled out, but the actual cause is still open.

- INT8 precision (10.6) is a promising algorithm-level result but has not been validated against real fixed-point hardware or a butterfly-stage-accurate FFT model — this is the natural next handoff to a silicon architect or FPGA test, not something further simulation alone can close.

- Whether 2 depth planes (dropping mid) are perceptually acceptable is still untested — 10.8 confirmed dropping mid saves real compute and isn't total loss, but the specific content-adaptive plane-count idea was retracted after a full-resolution retest reversed it. The perceptual verdict itself remains a question for a human or optical engineer, not simulation.

- Systematic audit needed, not yet done: two findings this revision (10.3's sequential-rendering result, 10.8's content-adaptive plane count) turned out to have been measured before this project's multi-seed and full-resolution standards existed, and neither had been retroactively re-checked until caught by chance. It is currently unknown what else in this document — particularly 10.2's plane-count figures and 10.4's color figures, both single-seed as far as is currently documented — carries the same risk. Flagged as a real, currently unresolved gap in this document's own rigor, not deferred by oversight.

- Standing rule, established this revision (10.7/A.9), reinforced three times now across two different mechanisms: any comparison of convergence speed between two conditions must verify both have reached true, plateau-confirmed convergence before being trusted, using a detection method anchored to the run's actual final value, not just local flatness. A fixed iteration budget flipped a result in both directions (a fake 64% win in 10.7, a fake catastrophic failure in 10.5); a local-window-only plateau detector separately produced false "instant convergence" positives even under a generous budget (10.5's independent-content validation). The local-window detector is retired as this project's default; the final-value-anchored method is the standard going forward.

- Warm-starting and scene-cut robustness have now been validated against genuinely independent content (10.5), closing that gap — but real motion-capture or recorded UI-interaction footage still hasn't been tested, only procedurally-generated stand-ins for it.

# 11. Closing Note

This revision's central lesson is procedural, and it echoes the one before it. v13 rebaselined the compute figure after an earlier number turned out to be unreproducible; that replacement — 434 FFTs/frame, ~156W, ~312× over budget, reported from a single seed — was itself treated as settled and written into the document as current. It wasn't settled. Rerunning it across five seeds surfaced genuine algorithmic variance (30–65 iterations to plateau, not measurement noise) that a materially improved plateau-detection method failed to explain away. The figure used throughout this document is now a seed-averaged range, not a point estimate, and single-seed convergence measurements are no longer treated as sufficient anywhere in this project.

The same suspicion, pointed at a different result, produced the opposite outcome. An unexpectedly clean finding — that INT8 precision matches float32 quality — survived three direct attempts to break it: a more realistic quantization scale in place of an oracle one, a full-resolution true-convergence retest, and a multi-seed check of an apparent side-effect that turned out to be the same seed-variance pattern caught elsewhere in this revision, not an INT8-specific penalty. That finding held. If confirmed on real hardware, it is a genuine ~3× lever on the compute gap (10.6); it is reported here as a promising algorithm-level candidate, not yet a confirmed one, pending exactly the kind of hardware or FPGA validation Section 9 already stages for.

Both outcomes came from the same discipline, applied evenly rather than selectively: treat every clean-looking number as a hypothesis until it has been actively attacked, whether the attack is expected to confirm it or break it. That discipline cost real time this revision — probably more than it saved on either single number — but it is the reason this document is more trustworthy now than at the start of it, in both directions: the number that got worse under scrutiny (compute), and the number that earned real support instead of resting on a stated assumption (INT8).

v15 added a third case: three deterministic phase-initialization strategies were tested against the seed-variance problem in 10.1; one of them — donor-seeding — looked like a 24–64% speedup at a fixed iteration budget, a result clean enough to be genuinely tempting to write up as-is. Rerunning it to true, verified convergence on both sides reversed the finding entirely: no effect in one content direction, a real loss in the other. The lesson wasn't really about phase initialization — it was that a fixed iteration budget can flip the sign of a comparison, not just its size, whenever the two conditions being compared don't finish at the same rate. That became a standing rule for this project (10.9, Appendix A.9).

This revision tested that rule against a second, independent case, and it held again — in the opposite direction. The original scene-cut finding in 10.5 (severe negative transfer at small scale, warm-start never converging after an abrupt content change) had never been checked against true, plateau-verified convergence; it used the same kind of small fixed budget that produced v15's false win. Retested properly across five cut pairs at two severities and both resolutions, the catastrophic-failure result did not reproduce once. The same underlying error — trusting a result before confirming both conditions actually finished — had this time manufactured a fake disaster instead of a fake win. Two independent cases, in opposite directions, is a much stronger basis for a standing rule than one: this is no longer treated as a one-off catch in this project, but as a property of how these measurements fail if not made carefully.

v17 closed the remaining gap flagged in both of those investigations — every result so far had been built from one base scene, varied by shift or permutation. Retesting against genuinely independent, procedurally-generated content surfaced a third, different failure mode under the same general rule: not a budget too small, but a detection method too narrow. The project's local-window plateau detector reported warm-start converging at exactly iteration 5 for nearly every frame in a motion sequence — a number suspicious enough to investigate before trusting. It turned out to be a structural floor of the detector itself, not a genuine finding; a spot-check found two more silent false positives it had already produced elsewhere. A fixed budget and an under-specified detector turn out to be two different ways of making the same mistake — trusting that a measurement finished before confirming it actually did — and this project has now caught both. The local-window detector is retired; a single, final-value-anchored method is the standard going forward. The underlying results, once measured honestly, validated rather than overturned everything already believed about warm-starting: genuinely helpful on independent content too, never catastrophic, but a real, favorable-on-balance effect rather than a uniform win.

This revision's catch was smaller but the same shape, applied to a new question rather than an old one. Testing whether dropping the middle depth plane loses less content when little is actually happening there produced a result opposite to the hypothesis on first read. Rather than accepting the surprise or dismissing it, the two content profiles being compared were checked for whether they were actually comparable — and they weren't: one had been correctly measured over a small masked region, the other over the full frame, and the two profiles had different inherent reconstruction ceilings to begin with. A normalized metric controlling for both reversed the finding back to the expected direction. Neither the raw nor the normalized number was treated as automatically correct — both are reported together, with the normalization's own assumption stated plainly, rather than quietly presenting only the number that confirmed the hypothesis.

This revision's two remaining catches are the most consequential yet, not because the mechanism is new, but because of what they were found in. 10.3's sequential-rendering finding — that sparse content shows no benefit from sequential depth-plane rendering — has been in this document since its earliest surviving revision, and was the flagship example this document's own closing note repeatedly cited as proof of its rigor. It had never been checked against the multi-seed standard adopted in v15, because it predated that standard and nobody had gone back to re-verify it. Retested with 25 independent seed pairings, the finding reverses: sparse content shows a small, real, consistently positive advantage after all, not zero. Separately, 10.8's content-adaptive plane-count candidate — proposed only one revision ago — was retracted outright after a full-resolution retest flipped which content profile actually benefits more, the same small-scale-to-target-resolution reversal this document has now documented four separate times (10.3, A.9's donor-seeding, A.10's scene-cut negative transfer, and this one).

Neither catch came from a planned audit — both surfaced because someone went back and applied the project's current standards to an old result on their own initiative, not because a gap was flagged and scheduled. That is arguably the more important finding than either individual reversal: this document's own rigor has a real blind spot, in the form of every finding that was measured before its current standards existed and has not yet been re-checked against them. A systematic pass identifying which figures carry this risk is flagged in 10.9 as a genuine, currently open gap in this document's own process — not a hypothetical one, since it has now caught two real, previously-unnoticed errors on the first attempt.

The working discipline stays the same as every prior revision: state the physics and the measurements plainly, name what doesn't close yet, and let the specificity of the question stand in for credibility that would otherwise have to be earned by pretending the gaps — or the process failures — aren't there.

# Appendix A — Investigation & Process Log

This appendix preserves the detailed history behind the current findings in Section 10: what was tested, what broke, what was fixed, and what remains genuinely unresolved. Nothing here changes the conclusions stated in the main body; it's the audit trail behind them, kept for anyone who needs to reproduce, extend, or challenge a specific number.

## A.1 Compute estimate — how the figure evolved

The original closed-form estimate (4.4) assumed 14 FFTs/frame at 1.49 GFLOP each. A small-scale (256×256) synthetic-target test found a modest quality plateau at 84 FFTs/frame — already 6× the original assumption, but itself later found to be an artifact of a fixed iteration cutoff borrowed from an even smaller test, not genuine convergence. Profiler-measured per-FFT cost at the padded array size actually computed came in at 6.5 GFLOP, not 1.49 — a further ~4.3× correction, independent of the FFT-count correction. A genuine full-resolution, plateau-auto-detected convergence sweep replaced both informal estimates with a single measured figure. That sweep's own reported result (696 FFTs/frame, ~250W) is the figure addressed in A.4 below — it could not later be reproduced, and the current baseline (434 FFTs/frame, ~156W, reported in 10.1) is the figure that regenerates from the current codebase.

## A.2 Amplitude/intensity metric correction

GS's internal optimization target is amplitude (square root of intensity), and its internal per-iteration convergence history correctly compares against that scale. Every externally reported "final quality" figure in earlier revisions of this document, however, compared reconstructed amplitude directly against raw, un-square-rooted intensity — a real unit mismatch traced back to the first simulation script written for this project. A corrected metric (squaring reconstructed amplitude before comparison) was implemented and used to systematically re-verify every prior finding:

| Finding | Before fix | After fix | Conclusion changed? |
| --- | --- | --- | --- |
| Seq-vs-sim, 4,700×2,700 (target res) | ~0.0 dB | ~0.0 dB | No |
| Realistic content, dense plane | +3.0 dB | +2.1 dB (later +2.3 dB at true convergence) | No |
| Color: native/reuse/joint | 9.3 / 7.1 / 8.2 dB | 6.8 / 5.8 / 6.3 dB | No |
| Gap 7 plane-count | 16.7 / 14.0 / 13.2 dB | 14.6 / 11.9 / 11.5 dB | No |

Note: No row in the full re-verification table changed sign or reordered a ranking. Absolute values dropped across the board (intensity = amplitude², so a fixed amplitude error looks larger once squared); most deltas between conditions shrank by roughly 20–30%. The end-to-end compute measurement (A.1/A.4) was never affected by this bug — its convergence detection used the internal, self-consistent amplitude history throughout.

## A.3 Full-resolution border artifact — investigation record

A faint axis-aligned grid pattern appeared in full-resolution reconstructions, absent at small scale. Five hypotheses were tested directly and ruled out: padding factor, aspect ratio, floating-point precision, the frequency-domain evanescent cutoff formulation, and the propagation math in isolation (clean with no GS loop). The working theory — uncontested energy accumulating at frame borders, where GS's amplitude constraint is weakest — motivated an edge-taper fix, tested and found to give a small quality gain (+0.2–0.3 dB) without visually resolving the artifact.

Note: STATUS: closed as non-blocking, not closed as explained. The root cause is not fully diagnosed. The 10.3 conclusion (no measured sequential-rendering advantage for sparse content at target resolution) is treated as confirmed regardless, on the strength of two independent robustness checks that both produced the same result under direct perturbation.

## A.4 Non-reproducibility of the original end-to-end compute figure

Only three git commits exist for this project's history (first_commit, cgh_0.01, cgh_0.02); most intermediate development, including the code state that produced the originally reported 696-FFT/250W/500× figure, lived only on disk and was overwritten by subsequent edits without being committed. The Compute/ directory containing the original output log was not committed until the most recent commit.

A controlled test — running the last committed pre-fix code state (commit 572270e) against the current target/propagation files — produced 37 iterations, not 58, under otherwise identical, deterministic conditions. Since the math is deterministic, this is not measurement noise: the exact code state that produced the original log genuinely no longer exists anywhere, committed or not.

Note: This is not an error in the original number — it was a real result of some real code at some point. It is a reproducibility failure: that code state was never a durable artifact. Process changes adopted going forward: commit meaningful experiment states as they occur rather than only at named milestones, and pin the Python environment (pip freeze or equivalent) alongside any output log that a document figure depends on.

## A.5 Convergence-history indexing bug

A separate, smaller bug was found and fixed during the reproducibility investigation above: the per-iteration history list scored each iteration's incoming phase quality rather than its outgoing (post-correction) quality, making every "final quality" figure in this document technically the quality after n_iters−1 corrections rather than n_iters. Fixed in both codebase copies (Demo/ and Neural-prior/).

Note: Re-verified with a controlled old-vs-fixed comparison: the fix produces an exact one-index shift in the history array and nothing else. Re-running the compute convergence sweep with this fix changed the plateau point by one iteration (37→36) and the FFT count by 10 (444→434) — within the range of change 10.1d-style re-verification already treats as "no conclusion change." Does not overturn any finding in this document.

## A.6 Content-generation bugs

- Font portability: text targets were initially rendered via a hardcoded OS font path that doesn't exist on most machines, silently falling back to a near-invisible default font and producing a misleadingly high PSNR against an effectively blank target. Fixed by rendering through the plotting library already used throughout the project, which bundles its own font.

- Sparse-content PSNR: whole-frame PSNR on a mostly-empty target (an icon or caption on a large black background) is dominated by trivially correct background pixels. A masked PSNR, computed only within the content's bounding box, is required for any sparse target; dense content does not need this correction.

## A.7 Seed-variance in convergence, and a failed detector-improvement attempt

The 10.1 compute baseline (itself a replacement for the unreproducible figure in A.4) was initially reported from a single seed: 434 FFTs/frame, 156W, 312× over budget. Rerunning the same full-resolution, true-convergence measurement across 5 seeds found plateau iteration ranging from 30 to 65 — a spread wider than the single-seed number's entire margin of apparent precision — while final quality stayed tight (11.47–12.00 dB, 0.53 dB spread).

A materially improved plateau-detection criterion was implemented specifically to test whether this variance was a measurement artifact: instead of triggering on a single threshold crossing, it required quality to hold within a margin for several consecutive iterations against an averaged reference. Tested directly against the same five convergence histories, it reduced the spread from 35 iterations to 34 — under 3%, no meaningful improvement.

Note: This rules out a sloppy detector as the source of variance. The instability is a genuine property of the algorithm's sensitivity to random initialization — some starting phases have a short path to the target quality, others take a substantially longer route to the same destination — not a measurement problem. The 5-seed average (552 ± 198 FFTs/frame, 199 ± 71W, ~397× over budget) is used as this document's baseline going forward. Single-seed measurements are no longer treated as sufficient for any figure quoted in this document; see 10.5's warm-start speedup for a case where a seed re-check was performed and the original single-seed figure held up instead — the fragility found here is not universal, but it isn't detectable without checking.

## A.8 INT8 quantization — stress-testing sequence

An initial test (16-iteration fixed budget, 512×512, quantization scale recalculated exactly from the true field range each iteration — an "oracle" scale no real ASIC would have access to) found essentially no quality loss at 8 bits versus float32. A result this clean, on a first pass, against a stated assumption in Section 4.5, was treated as a hypothesis to attack rather than a finding to report.

- Oracle-scale check: the oracle quantization scale was replaced with a static, ASIC-realistic scale — calibrated once and reused every iteration, rather than recomputed from the true field range each time. Only 4-bit was materially affected (−0.35dB → −1.62dB, sensible given 4-bit's minimal headroom); 8-bit was unaffected (+0.05dB vs. −0.02dB, within noise).

- Full-resolution, true-convergence retest: rerun at the document's actual 4,700×2,700 target resolution, run to true convergence rather than a small fixed budget. 8-bit still matched float32 (+0.01dB); 4-bit showed real degradation (−1.06dB) and plateaued almost immediately (iteration 4) — a mechanistically sensible signature of a quantization step too coarse to extract further detail from additional iterations.

- Convergence-speed side-finding, retracted: an apparent pattern of 8-bit needing more iterations to plateau than float32 was tested directly across multiple seeds and found to be the same seed-dependent plateau variance documented in A.7, not an INT8-specific effect — float32's own plateau iteration varies by up to 35 iterations by seed alone, which dwarfs any float32-vs-8-bit gap observed.

Note: The finding — 8-bit quality matches float32, 4-bit does not — held up under all three attempts to break it. It remains algorithm-level, boundary-only quantization emulation (rounding is applied where a real ASIC datapath would round, but does not model error accumulating inside a single FFT's internal butterfly stages) — not a hardware-validated result. See 10.6 for the current status and what would be needed to confirm it.

## A.9 Initialization strategy — full investigation record

Motivated directly by A.7's seed-variance finding: if random initialization causes genuine, unpredictable iteration-count variance, does a deterministic, informed starting point avoid it?

Flat-phase backprop: constructed by averaging each target plane's backward propagation with zero (flat) phase, once, before any GS iterations — reusing the same math already inside the GS correction step. At 512×512 with a generous fixed budget (60 iterations, not separately re-verified as true convergence — see caveat below), this underperformed even a naive zero-phase baseline: 25 iterations to plateau vs. 23 for zero-phase, both worse than random's average of 20, with backprop reaching the lowest final quality of the three (13.17 dB vs. 13.31 and 13.52). Determinism was verified directly (identical results across different seed arguments) before this was trusted as a real, not spurious, result.

Low-resolution coarse-to-fine seeding: a fast low-resolution solve of the same target, upsampled as the full-resolution initial phase. Tested at two low-res budgets (10 and 30 iterations). Neither beat plain random initialization on total FFT cost (10-iter seed: ~596 average total FFTs; 30-iter seed: ~675 average; random baseline: ~550 average). The 10-iteration variant additionally introduced a new failure mode: a half-converged low-res guess committed the full-resolution solve to a specific wrong structure, in one case running slower (68 iterations) than random's own worst measured seed (65) — a partially-converged guess carrying false confidence turned out worse than no guess at all.

Cross-session donor-seeding: a phase pattern already converged from unrelated (donor) content, reused as a fixed default initializer, tested in both content directions (realistic-content donor seeding a synthetic target, and the reverse). At a fixed 80-iteration budget, this looked like a substantial, consistent win — 24% fewer iterations in one direction, 64% in the other, with multiple independent donors all helping. This result was treated with the same suspicion as the INT8 finding in A.8 before being trusted, and rerun to true, plateau-verified convergence (200 iterations, confirmed flat on both conditions in both directions):

| Direction | Random, true convergence | Donor, true convergence | Verdict |
| --- | --- | --- | --- |
| Realistic donor → synthetic target | 40.2 iters, 11.18 dB | 41.5 iters, 11.19 dB | No real effect — the apparent 24% speedup was entirely a fixed-budget artifact |
| Synthetic donor → realistic target | 94.2 iters, 12.08 dB | 108.7 iters, 11.90 dB | Reversed — donor-seeding needs 15% more iterations and costs ~0.18 dB |

Note: This is the most consequential result of the three, not because donor-seeding failed, but because of how it failed: a comparison that looked like a clear, direction-consistent, multi-donor-validated 64% win reversed into a real loss purely because the losing condition (random init, Direction B) hadn't finished converging within the 80-iteration budget used for the initial comparison. Both conditions needed to be run to true, individually-verified plateau — not a shared fixed budget — before the comparison meant anything. This is now a standing rule for this project: no convergence-speed comparison is trusted without verified plateau on both sides. It is applied going forward, not retroactively enforced on the flat-phase-backprop and low-res results above, which predate this discovery and used generous-but-unverified fixed budgets — those two results are not known to be wrong, only not yet re-checked against the same standard.

Note: Taken together, all three initialization strategies tested — structurally different in mechanism — failed to reduce or avoid the iteration-count variance from A.7 once properly tested. Current interpretation: that variance is a structural property of this algorithm's search landscape, not a gap that a smarter starting point closes. See 10.7 for the main-body summary.

## A.10 Scene-cut robustness — full retest record

The original scene-cut finding (severe negative transfer at 512×512, partial reversal at full resolution) was reported using a small, fixed iteration budget (30 iterations) — the same category of measurement that produced v15's donor-seeding false positive. Per the standing rule established in A.9, this was retested with true, plateau-verified convergence (generous 100-iteration budget, sustained-hold detection) across five scene-cut pairs at two severities: moderate (a non-wrapping pad-crop spatial shift of the same base content) and severe (a full permutation of which content sits at which depth plane, i.e. genuinely unrelated content per plane, not just spatially displaced).

| Pair | Severity | 512×512 cold range (avg) | 512×512 warm | Full-res cold range (avg) | Full-res warm |
| --- | --- | --- | --- | --- | --- |
| S0 → S1 (shift) | moderate | 15–26 (20.0) | 11 | 10–11 (10.5) | 9 |
| S0 → S2 (permute) | severe | 26–36 (31.0) | 26 | 10–23 (16.5) | 19 |
| S0 → S4 (shift) | severe | 14–27 (20.0) | 10 | 10–17 (13.5) | 7 |
| S1 → S3 (permute) | severe | 10–22 (17.7) | 23 | 13–16 (14.5) | 17 |
| S2 → S0 (permute) | severe | 14–31 (22.3) | 16 | 8–9 (8.5) | 13 |

Note: Zero of ten trials (five pairs × two scales) showed negative transfer — every single one converged, including severe content permutations more extreme than the original single test case. Four of five pairs beat the cold-start average at 512×512; only two of five did at full resolution, with the remainder converging normally but without a clear speed advantage — the same shrinking-at-scale pattern documented elsewhere in this project (10.3), applied here to the speedup magnitude rather than its sign.

Note: This is the second confirmed case, after A.9's donor-seeding reversal, of a fixed iteration budget producing a wrong conclusion from an otherwise carefully-run test — this time a false catastrophe rather than a false win. Two independent cases, in opposite directions, is treated as confirmation that this is a general property of convergence-speed comparisons in this codebase, not a coincidence specific to either investigation. All five pairs were constructed from one base content set (shifted or permuted), not independently sourced scenes — the zero-negative-transfer result is encouraging but not yet tested against genuinely unrelated real content.

## A.11 Independent-content validation, and a plateau-detector floor bug

Motivated by A.10's own open caveat: every warm-start and scene-cut result to that point was built from one base scene, varied only by shift or permutation. This retested both against genuinely independent, procedurally-generated content — a different random icon, word, and Gaussian-blob "photo" scene per trial, rendered independently rather than derived from a shared source — and a smoother parallax-motion model (per-plane random-walk velocity, nearer planes moving faster than farther ones) in place of the earlier constant-velocity shift. Still synthetic; no real motion-capture or recorded footage was available or used.

The initial run reported warm-start plateauing at exactly iteration 5 for nearly every frame, across all three motion sequences tested — a result uniform enough to be suspicious on its face. The cause: the project's default plateau detector (introduced in 10.7, a local-window method requiring several consecutive iterations to be mutually close in value) checks only local flatness, and 5 was the smallest iteration count the detector's own hold-window size made it structurally capable of returning — a floor in the tool, not a property of the data. A separate, final-value-anchored detector (confirming a candidate plateau's value actually matches where the curve eventually settles over a much longer run, rather than only that nearby values are close to each other) was used to cross-check, and a seven-frame manual spot-check from one sequence quantified the damage precisely:

| Frame | Local-window result | True final value | Verdict |
| --- | --- | --- | --- |
| 1 | 9 (quality 6.95) | 154 (quality 7.19) | False positive |
| 2 | 1 (quality 7.07) | 1 (quality 7.12) | Real — 0.05 dB gap |
| 3 | 1 (quality 6.96) | 1 (quality 7.04) | Real — 0.08 dB gap |
| 4 | 1 (quality 7.06) | 1 (quality 7.17) | Borderline — 0.11 dB gap |
| 5 | 1 (quality 7.03) | 1 (quality 7.08) | Real — 0.05 dB gap |
| 6 | 1 (quality 7.13) | 114 (quality 7.27) | False positive |
| 7 | 117 (quality 6.90) | 121 (quality 7.41) | Real, both agree |

Note: Two of seven spot-checked frames were false positives — cases where the reused phase's zero-correction quality happened to sit in a locally flat stretch of the convergence curve that was not actually near the run's true eventual value. This is a different failure mode from A.9's fixed-budget problem: the iteration budget here was already generous, and the detector still produced false convergence claims. The shared lesson across both: a convergence-speed claim is only as trustworthy as the method used to detect convergence, and both "ran long enough" and "detected the plateau correctly" need to hold, not just one. The local-window detector is retired as this project's default; the final-value-anchored method is adopted as the single standard for all future convergence-speed work.

Rerun consistently with the corrected detector throughout (not just spot-checked):

| Independent scene-cut pairs | Warm | Cold avg | Result |
| --- | --- | --- | --- |
| 0 → 1 | 3 | 87.7 | Beats |
| 1 → 2 | 84 | 125.3 | Beats |
| 2 → 3 | 21 | 80.7 | Beats |
| 3 → 0 | 77 | 80.7 | Beats (barely) |

Parallax-motion sequences (3 seeds, 8 frames each, cold and warm plateau iteration per frame):

| Seed | Cold per-frame plateaus | Warm per-frame plateaus | Win rate |
| --- | --- | --- | --- |
| 0 | 128, 136, 155, 71, 76, 72, 40, 104 | 128, 154, 1, 1, 1, 1, 114, 121 | 4/7 |
| 1 | 128, 119, 113, 72, 151, 31, 148, 87 | 128, 69, 106, 1, 1, 1, 1, 56 | 7/7 |
| 2 | 128, 137, 119, 155, 74, 131, 62, 53 | 128, 1, 146, 16, 1, 76, 1, 1 | 6/7 |

Note: 4 of 4 independent scene-cut pairs beat cold-start with zero negative transfer, confirming A.10's shift/permutation-based result on genuinely unrelated content. Across parallax motion, 17 of 21 real frame-to-frame comparisons (excluding each sequence's shared first frame) favored warm-start — 81%, with per-sequence rates ranging 4/7 to 7/7. This is the honest picture: favorable on balance, sometimes dramatically (several frames converge in a single iteration with confirmed near-final quality), occasionally a net cost, never catastrophic — validating, not contradicting, every warm-starting finding elsewhere in this document. The seed-to-seed spread in win rate is noted as a real, currently unexplained variability, not investigated further here.

## A.12 Two-plane dropoff — confound diagnosis and corrected metric

Motivated by 10.9's flagged question: would dropping the middle depth plane lose less content when little is actually happening there? Two content profiles were built for the middle depth specifically: CLOSEUP (sparse — a small icon-like element, masked to its own bounding region per the sparse-content methodology established in 10.6/A.6) and LAYERED (dense — a photo-like gradient filling the frame). For each, the 2-plane (near/far only) solution's phase was propagated to the dropped mid depth and compared against the real mid content, the 3-plane baseline's actual solved mid quality (ceiling), and a blank field (floor).

The initial, raw-dB comparison came back opposite to the hypothesis: CLOSEUP showed a LARGER gap-to-ceiling (3.82 dB) than LAYERED (2.71 dB), suggesting the sparse scene lost more by dropping mid, not less. This was investigated rather than accepted or dismissed. Two genuine, independent confounds were found:

- Measurement basis mismatch: CLOSEUP's sparse mid content was correctly masked to a small region for PSNR purposes; LAYERED's dense mid was correctly measured over the full frame. A small masked region and a full-frame average are not directly comparable quality measurements — different pixel counts, different score variance.

- Different inherent ceilings: LAYERED's own 3-plane baseline quality (6.55 dB) is lower than CLOSEUP's (8.94 dB) independent of the mid-dropping question — busy, textured content is inherently harder to reconstruct in this project's established pattern (10.2, Gap 7) — leaving LAYERED less floor-to-ceiling range to work with regardless of how much real content its middle depth carried.

A normalized metric — the fraction of each profile's own floor-to-ceiling range (blank field to full 3-plane solve) recovered for free by the 2-plane proxy — controls for both confounds:

| Profile | Ceiling | Proxy | Floor | Fraction of range recovered for free | FFTs saved |
| --- | --- | --- | --- | --- | --- |
| CLOSEUP (sparse mid) | 8.94 dB | 5.12 dB | 2.71 dB | 38.7% | +32.9% |
| LAYERED (dense mid) | 6.55 dB | 3.84 dB | 2.90 dB | 25.8% | +20.1% |

Note: Once normalized, the result reverses back to the original hypothesis: sparse-at-the-dropped-depth content recovers a larger share of its own achievable range for free (38.7% vs. 25.8%) and saves more compute when dropped (32.9% vs. 20.1% fewer FFTs/frame) — a coherent, physically sensible pattern. The normalization is a stated choice, not neutral ground truth: it treats each profile's dB range as equally meaningful per percentage point, which may not hold if two scenes have very different absolute ranges. Both the raw and normalized tables are kept in the main body (10.8) together, deliberately, rather than presenting only the number that confirms the hypothesis.

Note: This remains groundwork, not a perceptual verdict — see 10.9. Two content profiles is not a sample size that supports a general claim about content-dependent plane-dropping; the content-adaptive plane-count idea this suggests is logged as an untested candidate, not a validated recommendation.

## A.13 Two retroactive catches: 10.3's sequential-rendering reversal, and 10.8's full-resolution retraction

Both findings below predate this project's multi-seed and full-resolution standards (established v15–v17) and had never been retroactively re-checked against them. Neither was found by a scheduled audit — both surfaced from independently re-testing an old result on initiative, which is itself the reason a systematic pass is now flagged in 10.9 as an open item, not a completed one.

10.3 reversal: the original sequential-vs-simultaneous depth-plane finding — present in this document since its earliest surviving revision, and repeatedly cited in this document's own closing notes as a flagship example of its rigor — was based on a single seed pairing per condition at target resolution. Retested with 5 seeds per condition (25 independent seed pairings, simultaneous vs. sequential):

| Plane | Simultaneous avg | Sequential avg | Delta range (25 pairings) |
| --- | --- | --- | --- |
| Near (sparse) | 7.47 | 8.79 | +0.87 to +2.06 dB |
| Mid (sparse) | 10.53 | 11.21 | +0.30 to +1.21 dB |
| Far (dense) | 11.23 | 14.70 | +2.81 to +4.42 dB |

Note: Every one of 25 seed pairings, for every plane, is positive — the sign never crosses zero. The original test's near/mid deltas were small enough (sub-1–2 dB) to be genuinely indistinguishable from seed-to-seed noise in a single comparison; this is exactly the kind of effect a single-seed test structurally cannot separate from chance, regardless of how carefully that single test was otherwise run. This is a different failure mode from every prior catch in this project (unreproducible figures, fixed-budget artifacts, a detector floor) — those were errors in how a measurement was taken; this was a real effect too small for the measurement's own resolution, undetected because the required resolution (multiple seeds) wasn't yet standard practice when the original test was run.

Note: Metric caveat, stated plainly rather than glossed over: this retest used the amplitude-vs-amplitude convergence metric matching GS's own internal optimization target (the same family used throughout the smart-init, scene-cut, and independent-content experiments), not confirmed to be on the same absolute scale as the psnr_intensity-corrected metric behind the originally reported ~1.9–2.1 dB dense-content figure. The qualitative reversal (near/mid show a real, consistently positive effect) is trustworthy independent of this; the exact dB magnitudes above should not be assumed directly comparable to previously published corrected-metric figures without a dedicated scale-consistency check.

10.8 retraction: the content-adaptive plane-count candidate proposed in 10.8 (drop mid when its content is sparse, keep it when dense) was tested only at 512×512. Retested at full target resolution:

| Profile | Fraction recovered — 512×512 | Fraction recovered — full resolution |
| --- | --- | --- |
| CLOSEUP (sparse mid) | 38.7% | 21.8% |
| LAYERED (dense mid) | 25.8% | 28.4% |

Note: The ranking inverts, not just narrows: CLOSEUP leads at small scale, LAYERED leads at full resolution. This is the fourth documented case in this project of a small-scale finding reversing (not just shrinking) at target resolution, after 10.3's original sequential-rendering claim, A.9's donor-seeding speedup, and A.10's scene-cut negative transfer. The content-adaptive plane-count idea is retracted as a supported finding. It is not retracted entirely without residue: dropping mid saves real, consistent compute at both scales (~20–33% fewer FFTs/frame) and recovers meaningfully more than a blank floor in every profile and scale tested — only the claim that content sparsity predicts the SIZE of that recovery fails to hold up.

Note: Taken together, these two catches motivate the audit flagged in 10.9: this project's multi-seed and full-resolution standards were adopted partway through, and nothing has yet systematically checked which earlier findings — most plausibly 10.2's plane-count figures and 10.4's color figures, both currently documented as single-seed — carry the same undetected risk these two did.